Otto Wedding Venue- Building

Scope of Work

Demolition

- Demo of metal on exterior of existing 40 X 80 pole Building- Including Roofing

- Misc. Demolition of existing Electrical on walls

- Demo Two (2) Overhead Doors

Concrete

- New Concrete Footings and Foundation at 20 foot addition on the back of the building

- New Concrete Footings and Foundation at each 12’ x 100’ additions on each side of the existing building

- New Concrete Footings and Foundation at the 12 x 40 Vestibule Addition

- New Slab on grade concrete at all additions

- Concrete additions to get in floor heat

- New Concrete Stoops at Doors: 110B, 111A, 111D, 111C, 106A, 105A, 103, 101A, 100A, and 100B

- New Concrete at Trash Enclosure

- New Concrete at Mechanical Enclosure

- New Concrete Walk in front of Vestibule addition- Allowance for 5’ X 40’

- New Concrete Walk at Double Door 110B- Allowance for 12’ X 12’

- New Concrete Walk on West Side Addition- Allowance for 5’ X 100’

- New Concrete Walk on South Side Addition- Allowance for 5’ X 64’

Masonry- Stone

- New Stone Masonry on South Wall- See plan page A406

Steel

- New Steel Columns at Trash Enclosure and steel structure- Will attach Trex Decking or simar material.  Steel to be primed.- Figure Steel Columns with Base Plates

- New Steel Columns at Mechanical Enclosure and steel structure- Will attach Trex Decking or similar material.  Steel to be primed.- Figure Steel Columns with Base Plates

Lumber/Trusses/Metal Siding/Metal Roofing

- New Poles to continue pole Building at all additions.  Include base plates as all addition areas will receive footings and Foundations. **Please note on our website are pictures of the existing laminated posts.

- Please note we haven’t been able to find any plans for the existing building

- New Trusses for 20’ addition on south end of building

- New Trusses for Vestibule addition

- New Trusses for 12’ X 100’ addition on East Side of Building

- New Trusses for 12’ X 100’ addition on West Side of Building

- New Metal Roof existing building and South Addition

- New Metal Roof on Vestibule Addition

- New Metal Roof on 12’ X 100’ East Side Addition

- New Metal Roof on 12’ X 100’ West Side Addition

- New Metal Siding on East Side Addition- Figure 4’ Wainscoting

- New Metal Siding on West Side Addition- Figure 4’ Wainscoting

- New Metal Siding on Vestibule Addition- Figure 4’ Wainscoting

- New Metal Siding on South Side addition- Figure 4’ Wainscoting

- New Acoustical Liner Paneling on Ceiling of Reception Hall

- Trex Decking for Trash and Mechanical Enclosures

- New Insulated Metal Exterior Doors: 106A, 105A, 103, 101A, 110B- Primed

- New Solid Core Wood Doors and Wood Casing for Doors: 106B, 105B, 104, 102, 101C, 101B- Staining by Others

- Pocket Doors: 109D, 109C- Staining by Others

- Barn Doors: 107, 108A, 108B, 109A, 109B, 110A- Staining by Others

- Two (2) Vinyl Window Type B- 4’ X 5’

- Seven (7) Vinyl Windows Type A- 3’ X 2’

- Tyvek wrap

- OSB for Sheer Wall- Current North Wall is a sheer wall, with the additions, figure new South Wall to be a Sheer Wall

Rough Carpentry labor

- Framing of all additions- Pole Building- New Poles will sit on new foundation walls

- Interior wall framing

- Installation of Windows

- Installation of Tyvek wrap on all exterior walls

- Installation of Metal Exterior Frames and Doors: 106A, 105A, 103, 101A

- Installation of Backing

- Installation of OSB for Sheer Wall

Finish Carpentry Materials

- Trim for Windows

- Trim for Wood Doors

Alternate: Shiplap on walls of Bar 108

Architectural Casework/Cabinetry

- Vanities for Bathrooms

- 18” Solid Surface (Corian or Similar) Countertop with Brackets in Brides Prep Area

- Solid Surface (Corian or Similar) Vanity Tops with integral bowls

- Granite Bar Top

- Cabinets with Solid Surface (Corian or Similar) in kitchen

- Wood Veneer paneling for front of bar

- PLAM Shelves with Brackes and Coat Bar in Vestibule

Insulation

- Poly ceiling of Reception Hall and insulate with Blown Insulation to Building code

- Spray Foam insulation on Exterior Walls- Base Bid

- Alternate: Batt insulation on Exterior Walls

- Chutes in attic

- Poly Ceiling in Vestibule and insulate with Blown Insulation to Building Code

- Rigid insulation installed at new Foundation Walls

Gutters and Down Spouts

- Supply and install gutters and down spouts on Roof of Reception Hall

- Supply and install gutters and down spouts on Roof of East Addition

- Supply and install gutters and down spouts on Roof of West Addition

- Supply and install gutters and down spouts on Roof of Vestibule Addition

Storefronts

- New Aluminum Storefront Doors: 100A, 100B, 111A, 111B, 111C, and 111D

- Two (2) New Aluminum Storefront Windows type C at vestibule- 8’ X 5’

Roll up Door/Shutter

- Roll up Door/Shutter at Pass through from Kitchen to Reception Hall- Manual operation

Drywall/Taping

- Supply, Hang, and Tape Drywall: Office 106, Grooms Prep 105, Men’s Restroom 104, Women’s Restroom 102, Hall 103, Brides Prep 101, Reception Hall 100, Mech & Storage 107, Bar 108, Kitchen 109, Delivery and Storage 110, and Vestibule 111.

- Supply, Hang, Tape/Texture ceilings: Office 106, Grooms Prep 105, Men’s Restroom 104, Hall 103, Women’s Restroom 102, Bride’s Prep 101, Mech. & Storage 107, Bar 108, Kitchen 109, Delivery and Storage 110, Vestibule 111.

Acoustical Ceilings

- Acoustical Ceiling in Kitchen 109

Painting/Staining

- Paint walls/Ceiling in: Office 106, Grooms Prep 105, Men’s Restroom 104, Hall 103, Women’s Restroom 102, Bride’s Prep 101, Bar 108, Vestibule 111.

- Paint Exterior Doors: 106A, 105A, 103, 101A, 110B

- Stain Interior Doors/Frames: 106B, 105B, 104, 102, 101C, 101B, 107, 108A, 108B. 109C, 109A, 109B, 110A

Alternate: Shiplap on walls of Bar 108

Tile

- Quarry Tile with Coved Base in Kitchen 109

- Ceramic Tile with coved base in Bar Area 108

- Ceramic Tile with coved base in Women’s Restroom 102

- Ceramic Tile with coved base in Men’s Restroom 104

Carpet Tile/Rubber Base

- Carpet tile with Rubber base in Bride’s Prep 101

- Carpet tile with Rubber baes in Grooms Prep 105

LVT/Rubber Base

- LVT with Rubber base in Office 106

- LVT with Rubber base in Vestibule 111

- Rubber Base in Delivery & Storage 110

Sealed Concrete

- Sealed Concrete in Mech. & Storage 107

Polished Concrete/Rubber Base

- Polished Concrete and Rubber Base in Reception Hall 100

- Polished Concrete and Rubber Base in Hall 103

Alternate Wood Base:

- Alternate for Wood Base:  Office 106, Groom’s Prep 105, Bride’s Prep 101, Reception Hall 100, Reception Hall 111

Toilet Partitions:

- Plastic Toilet and Urinal Partitions in Men’s Restroom 104

- Plastic Toilet Partitions in Women’s Restroom 102

Grab Bars/Bathroom Accessories

- Grab Bars and Bathroom Accessories in Bathrooms as noted on plans/Code Requirements

Fireplace

- Supply and install large Gas Fireplace on South Wall

- Include Mantel and trim- Prefinished Wood

FRP

- FRP on walls: Mech. & Storage 107, Delivery & Storage 110, and Kitchen 109

Fire Protection Specialties:

- Fire Extinguisher in Kitchen

- Two (2) Fire Extingusher with cabinets in Reception Hall 100

Food Service Equipment- to include design drawings

- Food Service Equipment to include but not limited to:

- Gas Range

- Hand sinks

- Prep Sink

- Work Tables

- Glass Washer

- Coolers

- Freezers

- Stainless Steel Pass through counter at pass through

- Ice Maker

- Microwave

Window Blinds

- Window Blind on Office Window

Plumbing/Gas Piping- Design Build

- Supply and install all toilets/Urinals

- Connection of plumbing to vanities

- Water Heater

- Boiler for In-Floor Heat

- In-Floor Heat- See plan page noting locations

- Grease Duct for kitchen hood- If Required by health department

- Floor Drains

- Kitchen Sink

- Plumbing to Bar Equipment

- Plumbing to Washing Machine

- Mop Sink

HVAC- Design Build

- Furnace/AC Units

- Ducting above ceilings

- Diffusers/return air grills in Acoustical Liner Paneling

- Diffusers/Return Air Grills in Acoustical Ceiling in kitchen

- Diffusers/Return Air Grills in drywall ceilings

Electrical- Design Build

- Provide new Electrical Service- Please note there are stub ups provided already- See pictures on our website

- Provide lighting in all areas of space

- Provide outlets for TV’s in reception hall and bar area

- Provide 3 large fans in reception area

- Provide electrical wall heater in Delivery & Storage 110

- Provide Exterior lighting- Sconces on Building

- Provide Exterior lighting on Building- Sconces