Scope of Work

1530 3rd St NE

Rochester, MN  55906

6/29/2026 – RFP Scope

Lead Paint Assessment was Clear

Exterior

Front Step

- Remove wood steps and replace with precast or freestanding wood/trex composite

Painting

- None planned

Gutters

- Install 5” gutters and large downspouts - white

Concrete

- No work planned at this time

Interior

Painting

- Possibly painting new closet shelves/base shoe/cabinet duct box if needed

Flooring

- No work

Carpentry

- Box around new range hood duct above cabinet

- Add rod and shelf to 2 bedroom closets @ 60” AFF

- Add shelves to pantry closet

- Add shelves and a door to hall linen closet

- Add base shoe to kitchen cabinets

Plumbing

- Install hybrid heat pump style water heater sized for 3-4 people

Electrical

- Hard wire new water heater

HVAC

- Vent range hood to exterior / thru roof

- Add 1 baseboard return air in hallway with 7” duct back to Return Main Duct

- HVAC advise on perm dehumidifier and better balanced return and supply

- Run static pressure test

- Duct cleaning

Radon Mitigation

- not required (2.2)

Insulation

- Spray foam rim joists and seal up penetrations

- Seal off old water heater vent by glass block

- Attic insulation ok as is

- Air seal around any attic / ceiling penetrations

Provide Option Price for One Car Detached Garage:

12ft wide x 24ft deep on thickened edge slab

Connect to existing black top driveway

Electrical

Overhead door with electric opener

Interior and exterior lights

Steel side door with dead bolt hardware

Final Clean can be provided by First Homes

First Homes Action Items

To do

- Change furnace filter

- Clean ERV filter

- Liebenow foundation evaluation

- Install dehumidifier

- Lead assessment

- Test side door water proofing

- Sewer line camera inspection

- Jetter clean sewer line root intrusion

- Energy Audit

Level dryer

Replace duct with rigid good duct

Clean basement floors – sweep, hepa vac, mop

Tape R/A vents

Monitor side door leaks in basement – none so far 6/30

Weird soil smell and high PM2.5 levels

-evaluate after water heater and gutters

FH directed work

Block Foundation

- FH will work with Liebenow Waterproofing to repair foundation

- Tuckpoint larger cracks

Landscaping by FH

- Add seed/straw on bare spots

- Completed 6/19/26

Trees – Work Provided by Others working directly with FH

- Dead Pine Tree in front yard to be removed

- Completed 6/1/26