Scope of Work

2724 12th Ave NW

Rochester, MN  55901

6/29/26 RFP Scope

6/29/26 Revised back door scope

Waiting on Lead Paint Assessment

Exterior

Gutters

- Repitch gutters on north side for better capture off roof

- Change down spouts so front discharges to front yard and back discharges to back yard

- Evaluate need for 3rd downspout

Front Door

- Remove and replace main door with fiberglass/partial glazed door

- New hardware

- Reuse storm door

- Paint both sides

Back Door

- Remove and replace back door with basic steel entry door

- New hardware

- Reuse storm door

- Paint both sides

Painting

- Re-stain front of house T1-11 panels

- Re-paint garage service door exterior side only

- Re-paint house back door / fill holes with epoxy or bondo (paint both sides)

Concrete

- Replace driveway and approach with concrete

- Seal along new driveway/house foundation

Garage

- wrap service door and oh door jambs

- repair top panel on OH door

Interior

Electrical

- Replace interior back entry door light fixture sconce

- Replace painted outlets – anything that needs to be replaced has the cover plate removed

- Replace primary bedroom switch

- Replace dining room light switch

- Power for bath fan / install smart switch

- Verify service is adequate

- Power for radon fan

Painting

- Paint hand rail on stairs going to basement

Flooring

- Replace kitchen floor – provide option

Carpentry

- Replace (2) Compromised 2 x 4 roof supports in attic

Plumbing

- Replace washing machine water supply valves and hoses

- Replace softener and discharge line

- Replace vanity top on main level – cracked [24 x 16]

- Reuse vanity faucet

- Main level bathroom vanity drain line – replace accordion style pipe

- Replace drain tite plug in tee at clean out

- Demo dead leg gas line to dryer that is no longer needed

HVAC

- Demo Hood and Seal off old exhaust duct above laundry

- Vent Kitchen Microhood to exterior (use insulated duct) – go up through cabinet

- Install bath fan in main level bathroom 80 cfm above shower (use insulated duct)

- Duct cleaning

Radon Mitigation

- Test results 5.9

- Install active system

Insulation (HIP has already quoted this work)

- Spray foam rim joists and seal up penetrations

- Attic insulation touch up blow

- Air seal around attic penetrations: chimney, vents, etc

- Add baffles to every roof truss channel for better ventilation

Final Clean can be provided by First Homes

First Homes To do

- Change furnace filter – aprilaire 210

- Liebenow foundation evaluation

Replace shower head

Replace door chime

Replace duct with rigid good duct

Lead assessment – to be scheduled once scope of work is finalized

- Sewer line camera inspection – completed 6/18

- Energy Audit – completed 6/16

- Permits closed out

Fix sagging exterior metal soffit

- Check Lead Pipe Map – checked – ok

Put sign on furnace humidifier – do not exceed X % in winter

Tighten heat register screws

- Replace bath register with 6 x 12 white

Schedule window washing

Add lint sock to washer discharge hose

Check sprayer on kitchen faucet

Swap basement bathroom bath fan cover for cover with integrated light Broan

Window Screens Replacement

Cut volunteer trees and spray

Garage Service Door

- Replace garage service door hardware – bronze

- adjust door for proper latching

Back Door

- Replace window trim-FH (measure)

- Replace door hardware with bronze-FH

- Replace weather stripping-FH – materials on site

- Not cost effective

Attic

- Potential moisture / mold issue

- Having Service Restoration evaluated on 6/19

- Mold mitigation completed on 6/26

Foundation Work

- Fix egress window area and add drain

- Replace daylight windows

- Fill cracks in block/mortar

- Sealant at back patio and step areas

- Repair face pops

- Displacement repair at beam

- Provide weather proof cover to daylight window that is blocked off

- Reconnect window well on north side

Landscaping

- Trim shrubs at front

Trees

- Evaluate tree pruning and health